名字：艾哈迈德								      学号：1820171030

RFID安全防护体系的构建与应用

- What is an RFID

1.1 Brief History

RFID is short for ‘Radio Frequency Identification’ the word itself is compound of two parts, the physical part i.e. ‘Radio Frequency’ and the means of usage ‘identification’  which this technology was originally intended for. RFID is sometimes called as ‘The Oldest New Technology’. It was first used by the British Air force in 1940s to identify friendly aircrafts or non-friendly aircrafts in the IFF (identify friend or for) system [1]. Later in the 1960 companies started commercializing the use of Radio Frequency for the use of Anti-theft systems, which is still broadly used up till today.

1.2 Brief Introduction to the RFID technology

RFID technology provides the ability to distinctively identify objects using devices called transponders (tags) through a wireless communication medium. The technology is made up of three essential parts: a Radio Transponder (commonly called as Tag), a Radio Reader and Transmitter, and a Host

System. The RFID tag and reader of a specific system communicate on a specified radio frequency. When an RFID tag goes into the range zone of the reader (the tag is usually adhered to an Object or sometimes a person), the reader tells the tag to send its data to it (which is usually an ID number). After the reader collects the data from the tag, it sends the data to the RFID controller by a network

Connection (Ethernet, Mobile IP, etc.). The controller will take the information and use it depending on the

type of data it has received.

Figure 1. A typical RFID System

1.3 Types of RFID tags

RFID tags are basically divided into 4 types Active RFID tags[i], Semi-Passive RFID tags, Passive RFID tags, and Semi-Passive RFID tags.

1.3.1 Active Tags:

Active RFID tags comes with power supplied on board usually a battery. Passive tags have longer read range than passive tag because they have their own power supply. The drawbacks are that they are more expensive and bigger in size. Active tags send out signals which are encoded with their identifiers at regularly scheduled rate usually between 1 to 15 seconds (known as beacon rate) [2][3]. Active RFID tags can be read from a distance of 300 feet to more than 1,500 feet away, while passive RFID tags offer an effective range of less than 30 feet. Active tags can also be read while moving at 60 miles per hour or more, with near 100 percent reliability[4].

Figure 2 Active RFID tag

1.3.2 Passive tags:

Passive RFID tags don’t have any internal power supply. Which is why they cannot send any signals on their own. When brought inside a Rang of the Reader they use the incoming radio signals to power up their integrated circuits and transmit a response. Their Antenna must be able to receive power form incoming radio signals and transmit a response signal. They can be as smaller than a millimeter (Hitachi holds the record for the smallest RFID chip ever, at 0.05 mm × 0.05 mm[5]) and as large as a podcast.  They can be reactivated years after the manufacture, thus making their lifetime almost unlimited. Since they don’t have an internal power supply they are very cheap to manufacture. Passive tags can operate at: LF(124-135 kHz)-(< 50 cm),HF(13.56MHz)-(< 1 m),UHF(860-960 MHz, 2.4 GHz)-(< 10 m)

Figure 3 Passive RFID tag

1.3.3. Semi Passive/Active tags:

The Semi-Passive and Semi-Active tags both contain power supplies (battery) on board. The main difference is how the power supply is used. In semi-passive tags the batteries are only used to power the internal circuitry. The semi-passive tags still needs to be in the range of the Near Filed in order to absorb power for data transmission. The advantage of semi-passive tags is it has longer read ranges than passive tags because the energy they absorb from Near Field is fully used to transmit data only.  In Semi-active tags the batteries are used exactly the same as those in active tags; however, the battery energy will only be used to power the tags when the tags are being interrogated by RFID readers i.e. when the tags are inside the Near Field. The benefit of semi-active tags is that semi-active tags can last longer than active tags since the batteries will only be activated when the tags are being interrogated by RFID readers [2] [3].

The choice between Active or Passive RFID tag depends on the type of system that is to be made. For some cases it is best to use Active tags whilst for others passive is the best choice. Bellow I present a table of some of the common differences between the Active and Passive RFID tags.

Figure 4 Differences between Active and Passive RFID[6][7]

Range of operation of RFID tags

Figure 5. Typical Range of Operation [12][13][14]

1.4. Why Use RFID?

RFID Radio Frequency Identification is one of the leading technology in the field IoT i.e. Internet of Things. RFID may be viewed as a means of explicitly labeling objects to facilitate their “perception” by computing devices. RFID has seen an increasing swirl of attention in the past decade. One important reason

for this is the effort of large organizations, such as Wal-Mart, Procter and Gamble, and the U.S. Department of Defense, to deploy RFID as a tool for automated oversight of their supply chains. Advocates of RFID see it as a successor to the optical barcode familiarly printed on consumer products, with two distinct advantages the Unique Identification and Automation [8]. In fact RFID technology can do everything a barcode does and much more (Johanasson, 2004). Barcodes, being optically scanned, require a clear line-of-sight contact between the readers and the barcode, and therefore objects should be careful physical positioned in order to be scanned. In contrast, RFID tags can be read without any line-of-sight contact and without precise positioning. RFID readers can scan tags at rates of hundreds per second. For example, an RFID reader at a warehouse door can scan stacks of passing crates with high accuracy all at once.

The main form of barcode-type RFID device is known as an electronic product code (EPC) tag. An organization known as EPCglobal Inc. [9] oversees the development of the standards for these tags. Not surprisingly, EPCglobal is a joint venture of the UCC and EAN, the bodies that regulate barcode use in the United States and the rest of the world respectively [8].

We are already using RFID technology in our daily lives for long now. Examples include Proximity cards, Bank cards, ignition key of millions of automobiles, RFID as anti-theft detection, libraries.

RFID can be used to facilitate the modern man’s life in many other ways such as;

1.4.1.Smart home appliances

Imagine a home that will not open the door until you or someone you have authenticated is physically present there, a home where the refrigerator could tell you that your milk is expired, or you have 1 tin of beer remaining, the refrigerator might even automatically send the grocery list to home delivery services [ii].

1.4.2. Shopping

Shopping using RFID-enabled payment and transmitting receipts to their mobile phones or saving them at the database of the shopkeeper could let costumers return items without receipts. RFID tags would act as indices into database payment records, and help retailers track the pedigrees of defective or contaminated items.

1.4.3.Health care system

RFID could bring huge changes and ease in the field of Health Care. Researchers at Intel and the University of Washington [10] have used RFID to facilitate medication compliance and home navigation for the elderly and cognitively impaired. For example, an RFID-enabled medicine cabinet could help tell the physician if the medications are taken on a time. If transplanted into human bodies all medical records of a patient could directly be transferred to hospital.

1.4.4. Retailing

RFID technology has been long used for retailing. It is used mainly for product tracking and inventory management. One of the first companies to use it is Wal-Mart. In June 2003, Wal-Mart Corporation issued a mandates for its top 100 suppliers to adopt passive RFID tag to all the shipments to 3 of its centers in Texas. By using RFID technology Wal-Mart was able to annually save $8 billion.

1.4.5. Animal tracking

RFID technology has for long been used for livestock tracking. Using UHF RFID tags the livestock agency has seen a huge ease in tracking their livestock from birth to the slaughter house [16]. Not only livestock common pet owners have also seen the benefit of RFID in locating their lost pet. Wild life researchers have also been benefitted form RFID UHF tags in tracking, locating and studying the behavior of wild life[17].

2. Privacy and Security Problems while using RFID Technology

Just like all technology stuff RFID technology is also not secure from being maliciously used by some wicked people. In this part of the paper we will discuss the main Privacy and Security concerns and the solution to prevent RFID from being maliciously used by others. First we will discuss the privacy issues and we will try to explain why RFID privacy is important and why it should taken seriously.

2.1. Privacy and Security Issues

In a world where from your grocery list to your medical health status everything is somehow connected to RF technology privacy could be a very serious issue. The main privacy issues while using RFID technology are inventorying and tracking.

As you might know by now RFID tags respond to any reader in range without the permission and knowledge of the owner. Thus making clandestine scanning a serious problem. Most RFID tags transmit their ID numbers either in plain text form or in an encrypted form. If the other party is able to make a connection between the tag ID number and the owner’s personal information such as national id number or credit card number, thus giving the other party a chance to establish a link between other transactions of the same customer.

A serious threat arises when we use EPC tags. EPC tags are made to carry some specific information about the product it is adhered to [15]. Thus making it plausible to unintentionally tracking by other. Suppose a customer is carrying grocery in a bag and she is on her way to home, since the items she is carrying have EPC tags on them and EPC tags can when in range of a reader can be interrogated without her being aware of it. Thus the information of what she is carrying could easily be tracked by someone. These information could be of very value, through simple EPC tag scanning you could know her preference of wine, her taste of clothing, the medicine she just bought or the one she is carrying in her bag thus letting you know what diseases she or one of her close relatives has, the place where she usually goes for grocery buying and the time at which she bought those things thus letting you know where and when she’ll probably go for her next grocery buying and many more. As we know today very few of our daily items carry EPC tags so, the above mentioned problems are not to be bothered with yet, but it is not far when most of our daily bought items would bear an EPC tag on them.

Another major privacy issue using RFID is tracking of people, as discussed above RFID is very affective in tracking livestock, pets and wildlife unfortunately it could also be used to track human beings. A very straight forward way would sub-dermal implants, as mentioned above RFID tags could be small in size thus making them easily injectable into the human body. Other ways could be slipping RFID tags into someone’s wear without the knowledge of individual [18].  Some governments have also wanted to keep track of their citizens using RFID technology. The Driver license, and citizenship cards of some countries carry RFID allowing the governments to keep track of their citizen in all time without letting them know. Cards like driver’s license are carried by citizens in all times thus making it nearly impossible for them to escape from tracked by their governments, this is a serious issue since privacy is basic human right of all citizens of the United Nations according to the 1948 Universal Declaration of Human Rights[19].

2.2. Security Requirements

Before we start stating the prescribed solution we should first state what we meant by security and privacy and what are the major types of RFID attacks.

- Confidentiality

Confidentiality is assuring that only authorized entities can share and access system resources and data.

The confidentiality objectives within the RFID System include:

- All data within the system should be protected from unauthorized access

- The algorithms for creating a-IDs can not be reverse engineered from known a-IDs

- Communications channels within the system should be protected from unauthorized access

- The data on the RFID Tags should be protected from access by unauthorized RFID Readers.

- Integrity

Integrity is the assurance that data is complete, un-modified and authentic.

This principle introduces requirements to protect against the modification of system data and resources:

- RFID Tag data should be protected from unauthorized modification

- The RFID interrogation channel and subsequent RFID data trail between the RFID System components should be protected from unauthorized modification

- All data within the system should be protected from unauthorized modification

- The presence of multiple tags should not cause loss of system integrity

- Duplication of RFID Tags should be prohibited.

- Availability

Availability is the assurance that authorized entities are able to access resources when needed.

The following availability requirements are relevant to the RFID System:

- All system components are operational 24 hours a day, 7 days a week

- The presence of multiple tags should not cause system outage

- The presence of multiple readers should not cause system outage

- Data accessed from the back end enterprise system should be available to multiple authorized personnel at any one time.

- Non-repudiation

Non-repudiation is the assurance that a sender or recipient cannot deny data modifications and data transmissions.

The assurance of this principle allows for entities within a system to trust one another and trust data integrity. As such, we state the following requirements within this principle:

- Mutual authentication between the RFID Tag and RFID Reader should occur

- Mutual authentication between the RFID Reader and Middleware should occur

2.3. Types of attacks on RFID tags

Since some RFID tags are cheap hence there computational resources are limited, making them vulnerable to a verity of attacks.

2.3.1.Cloning/Spoofing

This type of attack you must have seen in most of the modern Hollywood movies. A daily use of RFID is using it in employee ID cards, which are also used as door keys. As many big companies now a days have thousands of employees, an easy way of identifying whether he/she is an employee, a visitor or some infiltrator who is trying to sneak in could be using RFID imbedded ID cards. However with the ease RFID has brought in identifying employees it also have brought huge risks if not implemented correctly. These tags could be cloned by an infiltrator without even letting the poor employee’s knowledge. Getting close to an employee’s id and reading it through a reader and then doing some malicious acts is enough for making the cone of a RFID embedded employee card.

As we mentioned many car keys around the world use RFID technology in their keys. This also could be cloned unfortunately. Researchers at John Hopkins University together with RSA Labs have demonstrated successfully cloning opening the car with a cloned key.

An attacker can also perform Replay attack with a succefully counterfeited tag.

2.3.2.DoS Attack

Researchers have shown that success full Dos attacks can be launched on the modern UHF RFID tags. The test was done at Edith Cowan University in Perth[20].

2.3.3. Modifying the tag data

Another attack could be the unauthenticated modifying of the data. Since data in a RFID system is transmitted through a wireless channel the Man-In-the-Middle Attack could be of serious damage.  Data tempering occurs when the integrity of the data stored is compromised, this risk is further compromised when the RFID chip has a writable memory. Lucas Grunwald a German computer scientist has noted that with the proper tools it is easy to modify the data on cheap RFID tags [21].

2.3.3.Disclosure Attack

An adversary identifies the secret information of the tag and the secret keys used in the communication to fully compromise the security of the protocol.

2.3.4.Traffic Analysis

This is another type of attack where the attackers intercepts some pieces of information from messages being sent and received in order to get hands on some useful information that could be used to either infiltrate the system  or get private information of the user.

3.Proposed Solutions

As mentioned above, passive tags could be very cheap to manufacture, this might be a positive point but making them very cheap means less functionality. The EPC tags that are to be seen as replacing the traditional barcodes, it is very necessary for them to be extremely cheap since they are to be adhered to most of the products. So experts have worked hard to propose ways making privacy and security possible while using cheap tags.

3.1. Killing a tag

One proposed solution is killing the tag. That is, killing the tag when then item is sold to a costumer. This is a simple and very straight forward solution to many privacy problems. The tag could still be beneficial in the production line and even when in the shop, once the item is sold to a costumer the tag’s kill command is triggered making the tag unresponsive to all further investigations by any reader. Once the tag is killed no signal is transmitted by the tag, giving the costumer complete privacy, however this would mean that the earlier mentioned benefits of smart home appliances is no more an option. Since the tag won’t transmit any information it wouldn’t be of any good for the smart refrigerator that could tell you when your milk is going to get expired or when you’ll be out of yoghurt cartons.

3.2. The sleeping command

As you might already know the problem with permanently killing the tag is that the is tag is of no use after it’s kill command is triggered. One solution to this problem is temporarily putting the tag into sleep. The tags can be put to sleep giving the costumer complete privacy, and when needed the it can again be activated by the wake command [22].

3.3.Password/PIN protection

Another solution could be password protecting the tag. That is tags would only transmit data when presented the right password. Tags would only send data to readers that it already knows, who will send them the right password, since unknown reader don’t know the right password, the tag will not transmit any data.

3.4.Faraday’s Cage (Physical Shielding)

Covering tag with materials that cannot be penetrated by radio signals is another solution for safety. Since communication medium between tag and a reader is Radio Signal, preventing the tag from the reach of Radio Frequency is a hardcore solution. But this solution is tiresome since human intervention is needed to take the tag out for it to be further used. This method is best for credit cards and passports. Manufacturers around the world have made special card and passport holders that prevent unnecessary interrogations from the embedded tags.

3.5. Jammers

In some cases radio signal jammers are preferred. Jammers are put next to tags that prevent any radio tags from interrogating the nearby tags [23].

3.6. Security Based on Hash functions

Since the transferred between a tag and a reader is a long string or a few strings. Hashing them is a pretty straight answer to security problems. A hash is a one-way function that converts a variable-length block of data into a fixed-length gibberish called a “hash code.” It cannot be reversed. A hash function known only to two parties provides two principles; it authenticates the sender and it provides integrity assurance for sent data.

Weis et al. describe a method [24]by which access control can be achieved via a one-way hash function lock. In this approach each hash-enabled tag will have a portion of memory reserved for a temporary metaID and will operate in either a locked or unlocked state. To lock a tag, a tag owner stores the hash of a random key as the tag’s metaID, i.e. metaID ← hash(key). To unlock a tag, the owner asks the metaID from the tag, looks it up the appropriate key in the back-end database and finally transmits the key to the tag. The tag hashes the key and compares it to the stored metaID. If the values match, it unlocks itself and offers its full functionality to any nearby readers.

3.7. Pseudonym Tag

Another solution giving tags pseudonyms. Each tag changes his id, and has n pseudonyms. On each scan it returns a different pseudonym. This way it Unauthorized tag tracking would be more difficult. Although attackers could repeatedly scan the same tag, thereby forcing it to cycle through all available pseudonyms. This can also be countered. Tag should release a new pseudonym only every five minutes. This way if a tag has 100 changeable pseudonyms it would require 500 minutes to retrieve all of it’s pseudonyms and keep a track of them. The tag id must be reprogrammable.

## 3.8. Randomized Access Control

This approach is similar to Hash Based Access Control. Randomized access control (Weis et al., 2004) could improve traceability problem. Beside a one-way hash function, tags also have a random number generator. Each tag will operate in either a locked or unlocked state. An unlocked tag may be locked with a simple instruction from a reader; no protocol is necessary. Tags respond to reader queries by generating a random value, r, then hashing its ID concatenated with r, and sending both values to the reader. That is, tags respond to queries with the pair (r, h(ID||r)). A legitimate reader identifies one of its tags by performing a brute-force search of its known IDs, hashing each of them concatenated with r until it finds a match. Although impractical for retailers, this mode is feasible for owners of a relatively small number of tags.

## 3.9. Hash Chain

Initially tag has initial information s1. In the i-th transaction with the reader, the RFID tag sends answer ai = G(si) to the reader, and renews secret si+1 = H(si) as determined from previous secret si, where H and G are hash functions. The reader sends ai to the back-end database. The back-end database maintains a list of pairs (ID; s1), where s1 is the initial secret information and is different for each tag. So the back-end database that received tag output ai from the reader calculates ai’ = G(Hi(s1)) for each s1 in the list, and checks if ai ?= ai’. The back-end database find ai’ = ai and return the ID, which is a pair of ai’.

The hash chain technique (Ohkubo et al., 2003) could renew the secret information contained in the tag. Thus, the private information of user could be protected, and traceability becomes impossible.

## 3.10. XOR based One-Time Pad

XOR based one-time pad scheme, by RSA Lab (Juels, 2004), needs only an XOR calculation, and so is very low cost. In this scheme, the reader (actually the back-end database) and the tag share a common list of random keys, and in some interactions they confirm that the partner has the common list. If the check passes, the tag sends its ID. This scheme is very low cost. However, this scheme requires several interactions between the reader and the tag. Moreover, the common list must be overwritten completely as needed to ensure security. These points may make implementation difficult.

3.11. Anti-Collision Algorithms

Tag collisions occur when a reader reads many tags in the field at the same time, tags simultaneously sends back the data. This often happens when a lot of tags have to read at the same time such as when reading many cartons at the door of a warehouse. Tag collisions can be prevented with the use of anti-collision algorithms that essentially “singularize” a tag from a population of tags. RFID systems operating at 915 MHz generally implement the silent binary tree-walking algorithm as a singulation technique.

Although anti-collision algorithms are helpful commodities, their implementation nonetheless introduces significant tradeoffs that can potentially lead to less efficient and more costly systems. These tradeoffs include:

- The speed which a tag may be read

- The range at which a tag may be read

- The bandwidth of the outgoing reader signal

- The bandwidth of the incoming tag signal

- Tag and Reader Cost

3.12. Human Protocols

Weis [25] introduced the concept of human computer authentication protocol due to Hopper and Blum, adaptable to low-cost RFID. Besides the mechanisms described above, the National Institute of Standards and Technology of U.S. drew up the guidance for securing RFID systems. This security controls divided into 3 groups that are management, operational and technical in which described the considerations and controls in detail. Besides these security controls, it also discusses the privacy considerations including the privacy principles, applicable privacy controls and some other recommendations. In addition, the experts have been paying more attention to legal concerns over

RFID data collection. For example, in 1998, the European Parliament enacted guidelines on information privacy called the "European Community Directive on Data Protection". Under the EU Directive, information can be collected and used

only if in some certain purpose and conditions. To determine the lawfulness of a data processing operation, the Directive also sets out a number of principles. In the practical implementation of RFID systems, 6 golden rules have to be obeyed. Not only European, many countries also have set up regulations to restrict RFID uses from tag production to data collection.

Here in this part of the document I present a table of RFID System Risks, their impacts and the countermeasures that could be taken to solve those problems. This table has been taken from RFID Security and Privacy White Paper of the United States.

| Individual Information |  | Industrial Espionage |  | Traceability |  | Theft |  | Counterfeiting |  | Physical Attacks |  | DoS |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Kill Command Feature | ○ |  | ○ |  | ○ |  | 〤 |  | ○ |  | △ |  | △ |
| The Blocker Tag | ○ |  | ○ |  | ○ |  | 〤 |  | ○ |  | △ |  | △ |
| Rewritable Memory | ○ |  | ○ |  | 〤 |  | 〤 |  | 〤 |  | △ |  | △ |
| Physical ID | ○ |  | ○ |  | 〤 |  | 〤 |  | 〤 |  | △ |  | △ |
| Hash-Based AccessControl | ○ |  | ○ |  | 〤 |  | 〤 |  | ○ |  | △ |  | △ |
| Randomized AccessControl | ○ |  | ○ |  | ○ |  | ○ |  | ○ |  | △ |  | △ |
| Hash Chain | ○ |  | ○ |  | ○ |  | ○ |  | ○ |  | △ |  | △ |
| XOR Based One-Time PadScheme | ○ |  | ○ |  | ○ |  | ○ |  | ○ |  | △ |  | △ |

4.Conclusion:

As we have already seen RFID technology has brought vast changes in the way we do things. It is the technology past that will revolutionize our future. It would have already been implemented in a lot of our daily life thing were it not for the privacy and security problems it poses with its ease. As we know that the technology is being in use since 1940s but as we can see it is still not used in many ways that RFID technology can be used. The reason is if we make it cheap it brings a lot of security and privacy issues with it. If made expensive then it is not worth the cost. Hence one should be careful before implementing RIFD technology in their product or business models. In this document we have discussed some of the major security and privacy problems we face while using RFID. However it is not the complete reference and many  problems have not been deeply addressed here, if one is interested, the reverences are given interested individuals can visit and thoroughly read the papers, the references are one of the most prestigious papers and research surveys done on the topic of RFID.

References

- https://www.rfidjournal.com/the-history-of-rfid-technology

- Banks, David Hanny, Manuel A. Pachano, and Les G. Thompson, RFID APPLIED. NJ: John Wiley & Sons, Inc., 2007.

- F. Thornton, B. Haines, Anand M. Das, H. Bhargabva, A. Campbell, and J. Kleinschmidt, RFID Security. MA: Syngress Publishing Inc., 2006.

- https://www.rfidjournal.com/the-real-world-of-active-rfid-tags-2

- https://www.engadget.com/2007-02-14-hitachis-rfid-powder-freaks-us-the-heck-out.html

- Swedberg, C. Iotera Develops Active RFID Tag with 4-Mile Read Range. Available online: https://www.rfidjournal.com/iotera-develops-active-rfid-tag-with-4-mile-read-range-2

- Rouque, P. Performance Analysis of Effective Range and Orienteation for UHF Passive RFID. In department of Electrical and computer Engineering; Air Force Institute of Technology: Dayton, OH, USA

- IEEE JOURNAL ON SELECTED AREAS IN COMMUNICATIONS, VOL. 24, NO. 2. RFID Security and Privacy: A Research Survey. BY ARI JUELS.

- https://web.archive.org/web/20071109151629/http://www.epcglobalinc.org/standards/tds/

- K. P. Fishkin, M.Wang, and G. Borriello, “A ubiquitous system for medication monitoring,” Intel Res., Seattle, Tech. Memo IRS-TR-03-011

- K. Fishkin and J. Lundell, “RFID in healthcare,” in RFID: Applications,Security, and Privacy, S. Garfinkel and B. Rosenberg, Eds. Reading, MA: Addison-Wesley, 2005, pp. 211–228.

- Atmel Applications Journal 2004

- ISO 14443

- ISO 15693

- https://www.gs1.org/standards/epcrfid-epcis-id-keys/epc-rfid-tds/1-11#:~:text=GS1's%20EPC%20Tag%20Data%20Standard,information%2C%20and%20tag%20manufacture%20information.

- https://www.rfidjournal.com/tag/agriculture-livestock

- https://www.researchgate.net/publication/221091426_Wildlife_and_environmental_monitoring_using_RFID_and_WSN_technology

- Glasser, Goodman, & Einspruch (2007), p. 105

- Peslak, A. R. (2005). “An ethical exploration of privacy and radio frequency identification” Journal of Business Ethics 59(4):327-345

- https://www.zdnet.com/article/rfid-dos-attacks-proven/

- Sixto Ortiz Jr. How Secure Is RFID? IEEE COMPUTER SOCIETY, Computer Archive Vol. 39, 2006, pp. 17-19.

- Juels, R. L. Rivest, and M. Szydlo, The Blocker Tag: Selective Blocking of RFID Tags for Consumer Privacy, Proceedings of the 8th ACM Conference on Computer and Communications Security, 2003, pp. 103-111.

- A. Juels, “RFID security and privacy: a research survey” Selected Areas in Communications, IEEE Journal, vol. 24, no. 2, pp. 381‐394, Feb. 2006.

- S. A. Weis, S. E. Sarma, L. Ronald Rivest, and W. Daiel Engels, Security and Privacy Aspects of Low-cost Radio Frequency Identification System, Proceedings of the 1st International Conference on Security in Pervasive Computing, 2003, pp. 201-212.

- S. A. Weis, Security Parallels Between People and Pervasive Devices, The 3rd IEEE Conference on Pervasive Computing and Communications Workshops-PERSEC'05, 2005, pp. 105-109. 24. P. Van Eecke, and

- http://citeseerx.ist.psu.edu/viewdoc/download?doi=10.1.1.175.9165&rep=rep1&type=pdf

- An Italian Company Merloni has built prototypes of smart home appliances.